Table of Contents

Running an EEG Experiment:	2

1. Preparing the data:	4

2. ConvertToHTK	6

3. NeuralFindEvent	7

4. NeuralGenOut	8

5.	Final Structure	9

6.	Upload to Server	10

# Running an EEG Experiment:

Before starting the experiment:

Caps and Electrodes:

- Make sure that the cap is not folded and it is stretched.

-  	Cz should be at the middle of the head, approximately 20 Cm from A1 and 20 cm from A2. (Shown on the picture)

Cz

A1

A2

- Add gel and check the Impedance (open g.recorder/tools/impedancecheck), put gel in the ground and channel 1 at first. If you could not get green impedances for all the channels, save a snapshot of the impedance check.

- If you couldn’t get green impedance, lift the electrode a little and put gel under it, then fix the electrode.

- Connect the references to the ears. References should have gel too.

To Subject:

- It should be clarified for the subject that: 1. He shouldn’t move. 2. He should keep the eyes open during the experiment. 3. He should try to blink between the sentences.

Hiamp Configuration:

- Add 60Hz Notchfilter, 0 to 200 Hz bandpass.

- Add channel 64 as reference.

During the Experiment:

- Open g. Scope block in Simulink model.

- Check the analog channels.

- Check the eye-blinking effect on EEG, check the alpha power increase when the subject closes the eyes or when he is tired.

Between the Experiments:

- Make sure the subject is still concentrated.

- Check the impedances again.

After the Experiment:

- Wash each electrode of the cap separately without separating it from the cap. All the electrodes, wires and the cap are washable.

- Fill out the readme file, remove the unnecessary recorded data, and put all of them on the server.

A simple simulink model

# 1. Preparing the data:

- Create a folder named “CUEEGxx”, xx is the subject ID.

- Put “CUEEGxx.xls” (information file) in the generated folder.

- The name of the raw data should be “Bxx_ …”, xx is the block number.

- Create a subfolder  “original”, put the raw data and stimulus order in “original” folder.

* ”stimorder” and “data” in the name of the .mat files are the cues of finding the data and stimorder, so remember to mention them in the name of your .mat files.

- Stimorder is a cell array containing the sounds names in the order that were played.

- Raw data ( for example: B1_data_12_12_2014_13_04_10_0000.mat) contains variable y(channel*time)

  - y(2:63,:) EEG data

  - y(64:65,:) references

  - y(66,:) analog channel -> recorded audio using hiamp

# 2. ConvertToHTK

Go to “CUEEGxx” folder and run ConvertToHTK(fs). The default Sampling rate is 2400Hz.

Or specify the original and destination folder:

# 3. NeuralFindEvent

generated evnt:

# 4. NeuralGenOut

Generated Out:

aud is the auditory spectrogram

resp is channel*time*trial

trial contains the corresponding indices of trials in evnt

befaft is the added silence before and after the sound

- sound doesn’t contain the silence, resp and aud does.

# Final Structure

# Upload to Server

Upload the subject file containing the above structure (Original, Processed, evnt, and out) to the server in the appropriate task folder:

q/nimalab/h1/data/EEG/taskfolder/CUEEGxx